Radiator TS-3 Angle Steam Trap

Category: Steam Trap

Type: Radiator

Model: TS-3 Angle

The radiator steam trap shall be balanced pressure thermostatic type, self adjusting to all pressures within it's operating range.  The body shall be of cast bronze with male union inlet connection.  The thermostatic element shall be of precision welded stainless steel construction, incorporating a hardened stainless steel valve plug.  The valve seat shall be stainless steel and thread into the trap body.  All internals shall be of stainless steel construction and totally inline renewable without breaking any piping connections.